Starting with STM32CubeIDE Starting with STM32CubeIDE

[Dmytro Lyzun] [GitHub: https://github.com/Dima1381]

[Starting work in STM32CubeIDE, writing a first program with target: STM32F103C6T6]

[Last Updated: 27.02.2025]

This tutorial shows how to use the STM32CubeMX tool to initialize the peripherals, build and generate your starting projects with the initialization C code using HAL libraries.

Here I will go through the all process from launching the IDE to building the program on target. In this section I am going to develop a program called "A flashing led code". The idea is that the led on the target will display some code by blinking a certain number of times at intervals.

You can find the result in the same directory on github (file header)

Prerequisites:

- You should already have installed STM32CubeIDE.

Hardware:

- STM32F103x (in my case STM32F103C6T6);

- Jumper wires: M-M;

- ST-LINK for STM32 (in my case ST LINK V2);

Create new STM32Project in CubeIDE

Click File->New->STM32Project

Picture 1. Create new Project.

Find and select your target:

Picture 2. Selected target.

Write name project and click “Finish”:

Picture 3. Setup Project.

Attention: If you get the following error, just log in.

Click Help -> STM32Cube updates -> Connection to myST:

Picture 4. Error with login

After that, let's move on to some settings:

- Click System Core -> SYS -> Debug: Serial Wire.

- Setup Pinout, for out led blink we will use Pin 13. Click on PC13 -> GPIO_OutPut.

- Click “Save” and after STM32Cude create project and generate some code.

Picture 5. Setting project.

We will working in next file :

- Core-> Src -> main.c.

There you can find an infinite while loop – this is our work area.

Before write code – we should setup hardware.

Connection Hardware

To connect the microcontroller and the programmer i used this circuit scheme:

Picture 6. Connect the microcontroller and the programmer.

After that, we connect ST-LINK to our host (PC).

We return to our workspace (while loop) in STM32Cube.

Picture 7. Main work area.

Writing code

[Idea]:

There is a free LED on the board, under the port number PC13 (photo below). The idea is that this LED should reproduce certain signals. The program will generate an int array of 5 values ​​in the range (0-10). Please note that the code can only contain numbers from 0 to 9, so the number 10 will be read as 0. Led PC13 - will blink each value from the array. That is, the array will store a certain code, for example: [1, 3, 10, 7, 1]. Then Led PC13 will blink first 1 time, then 3, 10, 7, 1. We will pause between each value (1-2 seconds).

Picture 8. The LED (PC13) we are working with.

Let's create a function that will take an array and a size, then loop through the array and reproduce each value (Reproduce means turning the LED on and off as many times as there are values ​​in the array). You need to declare the function, and place the function itself after the main function:

void blink_code(int *array, int size) { // Let's turn off the LED for 4 seconds to simulate operation. HAL_GPIO_WritePin(GPIOC, GPIO_PIN_13, GPIO_PIN_SET); HAL_Delay(4000); // Delay function in milliseconds for(int i = 0; i < size; i++) { for(int j = 0; j < array[i]; j++) { // HAL_GPIO_WritePin(<TypeOfPort>, <Number_of_Port>, value (0-1)); HAL_GPIO_WritePin(GPIOC, GPIO_PIN_13, GPIO_PIN_RESET);// set 0 - Turn off the LED HAL_Delay(500); HAL_GPIO_WritePin(GPIOC, GPIO_PIN_13, GPIO_PIN_SET);// set 1 - Turn on the LED HAL_Delay(500); } HAL_Delay(1500); // Deley after each value in array } }

Next, we move to the infinite loop and implement the main logic there.

srand(time(NULL)); while (1) { int array[SIZE]; for(int i = 0; i < SIZE; i++) { // Filling an array with random values int rvalue = rand() % 11; // rvalue in the range 0 to 10 array[i] = rvalue; } blink_code(array, SIZE); }

Declarations before the main function:

#define SIZE 5 void blink_code(int*, int);

Include next library:

#include <stdlib.h> #include <time.h>

Build Project

Attantion:

If this is your first time creating a project in STM32CubeIDE.

When trying to compile the project, you may get the following errors:

Picture 9. Some errors you may get.

Working with ARM controllers, which uses arm-none-eabi-gdb. This is dependent on libncurses.so.5

To resolve this issue, run the following command in the terminal:

- sudo apt-get install libncurses5

Next, we will compile the project, click Project -> Build Project.

Picture 10. Building Project.

Make sure, that there no one error after building:

Picture 11. Successful project compilation.

After building, click Run -> Run Configuration…

Picture 12. Run Configuration.

Set the following settings and click “Run”:

Picture 13. Create, manage, and run configuration.

Execution process:

Picture 14. Successful Execution process.

After that, the program is compiled and sent to the microcontroller.

You can find the result and the program code on GitHub